ZERO — Progress Report

24 June 2026   ·   Platform: Raspberry Pi 5 (8 GB), fully offline   ·   Author: Gregory Kimani

# 1. Objective

Build the first step toward a conversational humanoid: a device you talk to in English that replies in a natural human voice — running entirely on-device, with no internet.

# 2. What We Built Today

A complete, working offline voice assistant with an interface-first, config-driven architecture — every stage is swappable via config.yaml with no code changes.

| Stage | Engine | Status |
| --- | --- | --- |
| Wake word | openWakeWord (“Hey Jarvis”, ONNX runtime) | Working |
| Voice activity / endpointing | silero-VAD | Working |
| Speech-to-text | whisper.cpp (tiny.en) | ~2 s / utterance |
| Conversation brain | Ollama + Llama 3.2 3B | Working |
| Text-to-speech | Piper (Fish S1-mini planned for emotion) | Speaking |
| Audio I/O | Logitech BRIO USB mic in · Bluetooth speaker out | Working |

Key features delivered.  Multi-turn conversation mode (wake once, converse freely, sleep after 30 s of silence); streaming replies (speaks each sentence while the LLM generates the rest); and diagnostic tooling for audio devices, mic level, wake score, and TTS.

Problems solved.  openWakeWord install (switched to ONNX for Python 3.12/ARM); whisper model loading; silero VAD 512-sample chunking; audio-queue overflow; and whisper hallucinations on silence (VAD threshold + minimum-speech gate).

# 3. Result

A genuinely usable offline assistant: wake → natural back-and-forth → sleep, with ~2 s transcription and spoken replies through the Bluetooth speaker. The full pipeline is proven end-to-end on the Pi 5.

# 4. Why a GPU Is the Better Long-Term Host

The Pi 5 has no ML-usable GPU — every model runs on its 4 CPU cores. This is the root cause of our main limitation: latency.

| Factor | Raspberry Pi 5 (CPU only) | GPU host |
| --- | --- | --- |
| LLM reply (3B model) | ~9–12 s / turn (a few tokens/sec) | < 1–2 s (50–100+ tokens/sec) |
| Speech-to-text | ~2–6 s | ~0.2–0.5 s (5–20× faster) |
| Expressive TTS (Fish S1-mini) | Impractical on CPU | Real-time — unlocks laughs / sighs |
| Memory | 8 GB shared; can’t hold all models | VRAM + RAM hold all models at once |
| Sustained load | Thermal throttling | Designed for it |
| Model headroom | Small models only | 7B+ LLMs, vision, humanoid features |

Recommendation.  The Pi 5 is excellent for proving the system and for low-power, portable deployment — and it has succeeded at that. But the project’s defining goals (near-instant conversation and a genuinely human, expressive voice) are gated by compute the Pi cannot provide. A GPU host — or a hybrid where the Pi is the edge device talking to a GPU server — removes the latency wall, makes expressive TTS viable, and leaves room for the vision and larger-model work the humanoid roadmap will need.

# 5. Next Steps

- Enable the expressive voice (Fish S1-mini) once a GPU is available; benchmark real-time factor.

- Train a custom “Hey Zero” wake word to replace the placeholder “Hey Jarvis”.

- Optional: add a short mic-mute window after speaking to prevent speaker-into-mic echo.