TABLE 1 · PROCESS SET

| PROCESS / THREAD | RUNS ON | FUNCTION | RATE |
| --- | --- | --- | --- |
| MainThread | Pi 5 | The four-state conversation loop | Event driven |
| PortAudio callback | Pi 5 | Microphone capture, gain, resample, echo pause | 30 ms frames |
| CameraStream | Pi 5 | Frame grab only | 640 x 480, 30 fps requested |
| Eyes | Pi 5 | Tiered perception, colour, tracking, scene diff | Per frame, gated |
| Narrator | Pi 5 | Tier 2 scene narration via the VLM | 3.0 s, budget capped |
| SurpriseGate | Pi 5 | Prediction-error scoring into episodes | On world change |
| proactive | Pi 5 | Presence, curiosity, idle consolidation | 3.0 s tick |
| zero-control | Pi 5 | HTTP control plane | Request driven, port 8090 |
| EyesWebPreview | Pi 5 | Optional camera preview | Request driven, port 8008 |
| stt-spec | Pi 5 | Speculative transcription at first pause | Once per pause |
| stt | Pi 5 | Final transcription | Once per turn |
| recall | Pi 5 | Relevance search in memory | Once per turn, 300 ms cap |
| llm-stream | Pi 5 | Background generation | Once per turn |
| tts-producer | Pi 5 | Synthesis ahead of playback | Once per turn |
| bargein | Pi 5 | Interrupt monitor while speaking | Once per turn |
| compaction | Pi 5 | Rolling summary of trimmed turns | On trim, single flight |
| memory-save | Pi 5 | Per-speaker durable write, corpus append | Once per session |
| timer-<id> | Pi 5 | One per pending timer or reminder | Until fired |
| Whisper server | GPU node | Speech to text, large-v3-turbo | Port 9000 |
| Orpheus server | GPU node | Expressive speech, 3B via vLLM | Port 9100 |
| Vision server | GPU node | Depth, scene facts, VLM, embeddings | Port 8000 |
| Ollama | GPU node | Conversational model, gemma4 8B | Port 11434 |
| SearXNG | GPU node | Web search backend | Port 8080 |